# Configure RADIUS in Packet Tracer

Time required: 60 minutes

How to Create Screenshots: Please use the Windows Snip and Sketch Tool or the Snipping Tool. Paste a screenshot of just the program you are working on. If you are snipping a virtual machine, make sure your focus is outside the virtual machine before you snip.

- Press and hold down the Windows key & Shift, then type S. This brings up the on-screen snipping tool.

- Click and Drag your mouse around whatever you want to snip.

- Release the mouse button. This places the snip into the Windows Clipboard.

- Go into Word or wherever you want to paste the snip. Hold down CTRL, then type V to paste the snip.

## Lab Description

In this project, you’ll configure RADIUS on a new Packet Tracer network. Complete the following steps:

## Activities

- Open Packet Tracer. Save your project as Radius.

- From the Network Devices  Wireless Devices menu, insert a WRT300N wireless router.

- On the wireless router’s Config tab, Click LAN  set the IP address to 192.168.5.1/24.

- From the End Devices menu, insert a Laptop.

- On the laptop’s Physical tab, replace the Ethernet network module with a wireless module:

  - Click the laptop’s power button to turn off the laptop.

  - Drag the PT-LAPTOP-NM-1CFE card from the laptop to the MODULES pane. Zoom in if necessary. The card is labeled "FAST ETH0."

  - Drag the WPC300N module from the MODULES pane to the laptop.

  - Turn the laptop back on.

  - A wireless connection is automatically created between the wireless router and the laptop. If not, troubleshoot the problem.

- Let’s set up some security parameters on the wireless router:

  - On the wireless router’s Config tab on the Wireless page, select WPA2 authentication.

  - Set the RADIUS server’s IP address to 192.168.5.2.

  - Set the shared secret to networkplus

  - Make sure the encryption type is AES.

- Why do you think this is?

Click or tap here to enter text.

- To solve this problem, you need to create a RADIUS server:

  - From the End Devices menu, insert a generic server.

  - On the server’s Config tab on the FastEthernet0 page, set its IP address to 192.168.5.2/24.

  - What is the subnet mask that /24 represents?

Click or tap here to enter text.

  - On the server’s Services tab, on the AAA page, turn on the AAA service.

- In the Network Configuration section, add a new client with the following information:

  - Client Name: WRouter0

  - Client IP: 192.168.5.1

  - Secret: networkplus

  - ServerType: Radius

  - Click Add.

- In the User Setup section, add a new user with the following information:

  - Username: User1

  - Password: cengage

  - Click Add.

- Insert a screenshot of the server settings.

Click or tap here to enter text.

- Using a Copper Straight-Through cable, connect the server’s FastEthernet0 port to the wireless router’s Ethernet 1 port.

- Has the wireless connection been reestablished?

Click or tap here to enter text.

- Why do you think this is?

Click or tap here to enter text.

- On the laptop’s Config tab, on the Wireless0 page, change the following settings:

  - Authentication: WPA2

  - User ID: User1

  - Password: cengage

  - Close the laptop’s configuration window. The wireless connection should reestablish within a minute or two.

  - Notice that you didn’t configure the laptop’s user information on the wireless router.

- How did the router know to accept the laptop as a wireless client based on your username and password?

Click or tap here to enter text.

- Insert a screenshot showing the wireless connection between the laptop and the router.

Click or tap here to enter text.

- Save your Packet Tracer file.

## Assignment Submission

Attach this completed document and the Packet Tracer file to the assignment in Blackboard.