# Disassemble and Assemble a PC

Time required: 1 hour

NOTE: Availability of a non-working or older computer was specified in the class schedule.

An IT Support Technician should be comfortable with taking apart a PC and putting it back together. They should also be familiar with the parts of a pc. Even though laptops are more popular, it is good to start with the basics where it is easy to see and understand all the components.

Tools: In most situations, the essential tools you’ll need for the job are an ESD strap, a Phillips-head screwdriver, a flathead screwdriver, paper, pen, and smartphone/digital camera.

Note: Take a photo of the interior of the PC before you start, and of various connections and devices as you go. This will help guide you in reassembly.

- Follow the directions in the book.

- Using a lab PC designated to be disassembled, take a computer apart. It is not necessary to remove the processor/CPU from the motherboard, or the motherboard from the case. If you do, be very careful to support the motherboard and processor carefully as you remove them from the case.

- With the PC apart, identify all the parts, cables, and connectors for the instructor.

- Reassemble the system. Don’t replace the PC case panel until your instructor has inspected all cable connections. Turn on the computer and verify that all is working only if it is a functioning computer.

## Submission

In Class: The instructor or proctor will watch, monitor your progress, and grade this lab.

Online Photo Submission: You will need to obtain or borrow a PC that you can take apart without fear of damaging it. You should be able to obtain or borrow an older or non-working PC from a friend, family member, Craig’s List, or a pc repair shop.

- Insert digital photos or a youTube video of the steps in the outline of your process and insert in this document.

- Use Word Shapes and Text Boxes to label the power supply, DVD drive, memory, CPU and cooler, and any expansion cards.

- On the Insert tab, click Shapes.

- Click the shape you want, click anywhere in the workspace, and then drag to place the shape. To create a perfect square or circle (or constrain the dimensions of other shapes), press and hold Shift while you drag.

- You can type into a shape or add a text box.

- After you have completed the lab, click on one of the inserted photos. Use the Picture Tools Format tab, Compress Pictures button to compress the pictures using the pictured settings.

- Attach this document to the assignment.

Online Video Submission: You can record and post a YouTube video of your process. Include the URL of the video in your assignment submission.

WARNING: Don’t do this project on a production PC!

# PC Disassembly Outline

NOTE: For specific directions, use the instructions in the book. In some places, I have changed the order of disassembly and assembly.

- Plan and organize your work.

- Backup important data (if this is a customer computer).

- Power down and unplug system. Disconnect any cables from the PC.

- Press and hold the power button for a moment to drain the capacitors in the power supply.

- Open the case cover. 
Insert a photo of the inside of the case. This photo will guide you in the reassembly if your notes or drawing doesn’t make sense.

- Clip your static strap to the side of the case and wrap the other end around your wrist.

- Remove the expansion cards.
Insert a photo after this step.

- Unplug the power supply lines from the motherboard and other locations.

  - Unscrew the screws from the power supply and remove it from the case.

- Insert a photo of the motherboard.

- Unplug the rest of the wires from the motherboard, making a careful note of where they were connected.

- Carefully unseat the memory by releasing the little levers on each end of each memory chip.

- Remove the hard drives and the optical drives.
Insert a photo after this step with the parts showing.

# PC Assembly Outline

Use your notes and your photos to reverse the process. Look in the book for specific instructions.

Compare your photos with the assembled pc.

Power on the PC when you are done and confirm that it functions (If it was functional before you started this lab).

Attach this completed document to the assignment in Blackboard.