# Arpspoof MITM - Performing Man in The Middle Attacks with Kali Linux

Contents

Arpspoof MITM - Performing Man in The Middle Attacks with Kali Linux	1

Lab Requirements	1

Setup NAT for Lab	1

Man in the Middle (MITM)	2

1: View Local IP Address Information	3

2: Obtain the IP configuration from the Victim	3

3: Turn on Packet Forwarding in Kali Linux	4

4: Redirect Packets to Your Machine with arpspoof	4

5: Intercept Packets from the Router	4

6: Sniffing URLs information from victim navigation	5

7: Stopping the Attack	5

8: Summary	6

Time required: 60 minutes

# Lab Requirements

This lab can disrupt network communications on a production network. We want to do this lab in a completely virtual environment.

- Kali Linux VM

- Windows VM

- Both VM’s on a NAT network

# Setup NAT for Lab

We need to setup a NAT network with DHCP to isolate this lab from your production network.

- Oracle VM VirtualBox Manager  Tools  File  Network Manager.

- NAT Networks  Click Create.

- Right Click NatNetwork  Properties.

- Network CIDR: 10.10.1.0/24

- Click Apply.

- In Oracle VM VirtualBox Manager: Right Click each Virtual Machine  Settings.

- Network  Attached to: NatNetwork

- Each VM should have Internet Access on a private network.

# Man in the Middle (MITM)

Man in the Middle attacks are some of the most frequently attempted attacks on networks. They’re used mostly to acquire login credentials or personal information, spy on the Victim, sabotage communications, or corrupt data.

A man in the middle attack is the one where an attacker intercepts the stream of back-and-forth messages between two parties to alter the messages or just read them.

# 1: View Local IP Address Information

On Kali Linux: Run the following command in the terminal to find out the name of the network interface that you’re using. It is commonly eth0.

ip a

Insert a screenshot:

Click or tap here to enter text.

Find the IP of the Router you’re using.

ip route show

On the terminal you will be shown the IP of your network router.

Insert a screenshot:

Click or tap here to enter text.

# 2: Obtain the IP configuration from the Victim

You need to get the IP of your Victim.

Nmap is an open source tool to scan networks.

- Open a terminal.

- Do a quick scan of your local network. Substitute your network information.

sudo nmap -sn 10.10.1.0/24

- Insert a screenshot:

Click or tap here to enter text.

- On the Windows VM, go to a command prompt and confirm the IP address.

# 3: Turn on Packet Forwarding in Kali Linux

This is very important because if your machine isn’t exchanging packets, the attack will result in a failure as your internet connection will be disrupted. By enabling the packet forwarding, you disguise your local machine to act as the network router.

In Kali Linux, to turn on packet forwarding, run the following command:

sudo sysctl net.ipv4.ip_forward=1

# 4: Redirect Packets to Your Machine with arpspoof

Arpspoof is a Linux utility that lets you expropriate traffic to a machine of your choice from a switched LAN. Arpspoof is a simple tool to redirect network traffic on the local network.

Install arpspoof.

sudo apt install dsniff

To find your network interface name, use ip a The network interface name in VirtualBox will be eth0. This is the first ethernet adapter.

Use the following syntax to start intercepting packets from the Victim to your Router:

sudo arpspoof -i <Network Interface Name> -t <Victim IP> <Router IP>

This has enabled the monitoring the incoming packets from the Victim to the Router. Leave this terminal running.

Insert a screenshot:

Click or tap here to enter text.

# 5: Intercept Packets from the Router

You’re doing here the same as the previous step, except it’s just reversed. Leaving the previous terminal open as it is, open up a new terminal to start extracting packets from the Router. Type the following command with your network interface name and router IP:

arpspoof -i <Network Interface Name> -t <Router IP> <Victim IP>

We switched the position of the arguments in the command we used in the previous step.

You are now in the middle of the communication between the Victim and the Router.

Insert a screenshot:

Click or tap here to enter text.

# 6: Sniffing URLs information from victim navigation

You can also sniff out the website’s URL that our Victim often visits. The program we’re going to use is a command-line tool known as urlsnarf. It sniffs out and saves the HTTPs request from a designated IP in the Common log format. Fantastic utility to perform offline post-processing traffic analysis with other network forensics tools.

The syntax you’ll put in the command terminal to sniff out the URLs is:

sudo urlsnarf -i [Network interface name]

As long as each terminal is functional and you’ve accidentally not closed one of them, Kali will capture all http traffic. There are very few web sites that use http, most use https.

- On the Windows VM  go to www.vulnweb.com

- On Kali  the urlsnarf terminal should show traffic to and from vulnweb.com

- Insert a screenshot:

Click or tap here to enter text.

# 7: Stopping the Attack

Once you’re satisfied with what you’ve got your hands on, you may stop the attack by closing each terminal. You can use the Ctrl+C shortcut to go about it quickly.

In Kali  disable packet forwarding.

Type in the following command in the terminal:

sudo sysctl -w net.ipv4.ip_forward=0

# 8: Summary

NOTE: This is not a script, it is a summary of the commands used.

# Enable port forwarding sudo sysctl -w net.ipv4.ip_forward=1 # Spoof connection between Victim and Router # Note: Run this command in a new terminal and let it run sudo arpspoof -i [Network Interface Name] -t [Victim IP] [Router IP] # Same step but inverted # Note: Run this command in a new terminal and let it run sudo arpspoof -i [Network Interface Name] -t [Router IP] [Victim IP] # Sniff URL traffic of the victim # Note: Run this command in a new terminal and let it run sudo urlsnarf -i [Network Interface Name] # This should show traffic to and from www.vulnweb.com # Disable port forwarding once you're done with the attack sudo sysctl -w net.ipv4.ip_forward=0